Phase 1: Mastering Time and Sleep (The Foundation)

Before adding complex sensor logic or two-way communication, the microcontroller must prove it can sleep and wake up reliably without draining the supercapacitors.

Step 1.1: Replace the Delay with Deep Sleep. Swap your HAL_Delay(5000) with the Enter_Standby_RTC() function. Set it for a short interval (e.g., 10 seconds) just to verify that the STM32 wakes up, flashes its LED, sends a dummy LoRa packet, and successfully goes back to sleep.

Step 1.2: Implement the RTC Jitter. Once basic sleep is working, write the logic to add the +/- 15-minute random jitter to the sleep duration. This prevents your nodes from synchronizing and shouting over each other.

Phase 2: The Fuel Gauge (Protecting the Power)

The node must be self-aware of its energy levels to avoid "brownouts" (crashing mid-operation).

Step 2.1: Basic ADC Implementation. Configure an ADC channel to read the supercapacitor voltage via a voltage divider.

Step 2.2: Primary Voltage Check. Add the logic: Is Vcap > 3.8V? If yes, proceed. If no, immediately go back to sleep.

Step 2.3: The Inrush Safety Net. Turn on the boost converter and sensors, wait 50ms, and take a second ADC reading. If the voltage sags below 3.0V, abort the mission and go to sleep.

Phase 3: The Complete Node (Integration & Two-Way LoRa)

Now we tie the power logic, the sensors, and the radio together.

Step 3.1: Update the Payload. Modify your lora_packet_t struct to include a byte for the supercapacitor voltage (e.g., sending 38 to represent 3.8V).

Step 3.2: The Rx Window. Implement the strict timing sequence: Transmit (Tx) -> Put LoRa to Standby for exactly 1.0 second -> Open LoRa Receive (Rx) window for 200ms.

Step 3.3: ACK Handling. If an Acknowledgement (ACK) is received from the Gateway, or if nothing is heard after 200ms, shut down the radio and execute the deep sleep command.

1. WiFi Manager

2. MQTT Manager

3. JSON Builder

4. MQTT Publish Task

5. Cloud Dashboard

6. Database

7. TLS Security

8. OTA

9. Device Authentication

10. Remote Commands

fyp_user
Cjed5Tdva52JbA4